|  | Central Perk Cafe 123 Bean Street, Fort Collins, CO 80524 Phone: (970) 555-0112 ¦ Email: ops@centralperk.example ¦ Web: centralperk.example Document: SOP-IT-003 ¦ Version: 1.0 ¦ Effective: 2026-01-15 ¦ Owner: Operations Manager |
| --- | --- |

# Network and Wi-Fi Segmentation for Guest, Staff, POS, and IoT

## Purpose

Prevent outages and reduce risk by separating guest Wi-Fi, staff devices, POS systems, and IoT equipment. Maintain consistent performance and limit the blast radius of a compromised device.

## Scope

Guest Wi-Fi network

Staff device network (tablets, phones, laptops as allowed)

POS network (terminals, payment devices, printers)

IoT network (cameras, smart TVs, music system, environmental sensors)

## Roles and Responsibilities

| Operations Manager | Approves network policy and guest access rules. |
| --- | --- |
| MSP Network Engineer | Designs segmentation, firewall rules, and monitoring. |
| Store Manager | Ensures physical security of network gear and reports outages. |
| Camera Vendor | Maintains camera firmware and access method approved by policy. |

## Frequency

Firewall rule review: quarterly

Wi-Fi health review: monthly

Firmware updates: quarterly or per critical advisory

## Procedure

Create separate VLANs or SSIDs for Guest, Staff, POS, and IoT with distinct credentials.

Apply firewall rules: block lateral movement between VLANs by default; allow only required traffic (POS to payment processor, staff to internet, cameras to NVR/cloud).

Enable WPA3 where supported; otherwise WPA2 with strong passphrases and rotation schedule.

Disable UPnP and inbound port forwarding unless explicitly approved and documented.

Implement DNS filtering for staff and IoT networks; apply stricter category blocks on IoT.

Configure QoS to prioritize POS traffic over guest streaming.

Monitor uptime and performance; set alert thresholds for WAN loss and AP failures.

Document a quick-recovery plan: spare AP, known-good config backup, and rollback steps.

## Checklists

Monthly Review

AP uptime and client load reviewed

Guest network bandwidth limits validated

POS connectivity tests completed during low volume window

Unrecognized devices investigated and documented

Change Control

Change documented with reason and expected impact

Backup of network config taken before change

Change performed in maintenance window

Post-change validation completed and logged

## Records and Evidence

Network diagram and VLAN/SSID map

Firewall rule exports and change logs

AP configuration backup files

Device allowlist inventory for IoT

## MSP Value

Design and maintain segmentation aligned to PCI expectations.

Provide 24/7 monitoring and proactive remediation.

Reduce troubleshooting time with standardized configs and documented recovery.

Deliver guest Wi-Fi experience without risking POS stability.

## Revision History

| Version | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 1.0 | 2026-01-15 | Central Perk Ops | Initial release |